M/V JOHN P LAB

REGISTRATION:				CAPACITIES:

Classification		✠ A1, ✠ AMS		Rig Water		120,000 gallons

Builder		Service Marine Industries		Lube Oil			2,000 gallons

Year Built		1998			Potable Water		35,000 gallons

Dry Bulk		8,000 ft3 in 5 tanks

CHARACTERISTICS:				Liquid Mud		3,000 bbls in 6 tanks

Length			210’			Mud System		2 Segregated Systems

Beam			  48’			Reefer/Freezer		600ft3 each

Depth			  18’

Maximum Draft	 	  13.5’			ACCOMMODATIONS:

Deck Area		5,520 ft2		Berths/Staterooms	24/7 staterooms

Deck Dimensions		138’ x 40’					2 x 1 person

Deck Load Capacity	900 t/800 lt					4 x 4 person

Deck Strength		1,080 lbs/ft2					1 x 6 person

Deadweight		2,127 t

PERFORMANCE:				MACHINERY:

Fuel Consumption:				Main Engines:		(2) CAT 3516B

Max Speed Cruising	13 kts @ 186 gph					4,000 BHP total

Economy	  9 kts @ 94 gph		Propellers		2 x 90”; 4 blade SS

Standby/Genset		6 gph			Z-Drives

DP Mode					Bow Thrusters		800 hp DITA retractable

Pumping Rates:								Fully azimuth

Methanol	250 gpm @ 200’ head		Driven By		CAT 3412 generators

Water		750 gpm @ 200’ head					2 x 170 kW, 480 v 60 Hz

Fuel		750 gpm @ 225’ head

Dry Bulk	1,000 ft3/hour @ 80 psi

Liquid Mud	1,000 gpm @ 185’ head		ELECTRONICS:

Radar			(2) Furuno

SPECIAL EQUIPMENT:			GPS 			(1) Furuno

FIFI	1 x 2,500 gpm pump			VHF			(1) SEA-235 150 watt SSB

Deck	(1) ABS Cargo Control & Monitoring	Radio			1612C SSB Automatic Coupler

DP System	Kongsberg Simrad SDP01;	SSB			(4) ICOM IC-M502 VHF

Joystick; autopilot; mixed		EPIRB			(2) ACR Pathfinder3 Search & Rescue

manual mode			Radio			Radios, Scanning VHF features